Securacath Removal

Follow the instructions for the routine removal of a PICC with SecurAcath until the PICC has been removed and the lower part of the SecurAcath in the only item left in situ.

Assess the amount of pain at the exit site by moving the lower portion of the device slightly:          Is there acute pain?

Remove the SecurAcath by:                                      a: pinching the two sides of the device together until they meet and pulling with a fast swift pluck or b: cutting the device in half with a scissors and removing each nitinol anchor separately with a swift pluck.

Administer lidocaine 1% at exit site. For non-prescibers – use the specifically prepare patient group direction.

No

Administer a small amount of lidocaine around the exit site with a small orange needle – withdraw on the plunger initially to verify that the needle is not in a vessel.

No

Observe the skin close to the SecurAcath pins.  Is the skin overlapping the pins?

Yes

Yes